AWS and EC2 Instances

What is security group

- it is a virtual firewall works at ENI (instance) level.

- up to 5 security groups per EC2 instance interface can be applied.

- can only have permit rules, cannot have deny rule.

- And its stateful means the return traffic is automatically allowed

- if you allow port in inbound traffic then you cannot attach separate port in outbound traffic the port will be automatically allow both inbound and outbound traffic.

Base points

- it is operated at instance level.

- support allows rules only.

- stateful, return traffic is automatically allowed.

- applies to an instance only

Inbound (Incoming):
1. Traffic coming into your server
2. Controls who can access your instance

Outbound (Outgoing):
1. Traffic going out from your server
2. Controls where your instance can connect

Create a Instance Windows & get the access

1. create a instance

1.name----------------> Windows_Server-2016-English-Full-Base-2026.02.11

2.instance type------> t3. micro

3.security group----> ssh / http / RDP (3389)

4.create a instance

2.access the instance

1.connect the instance

2.go to RDP client

3.click on get password------> browse key pair-----> decrypt password ---> copy

4.open RDP in desktop

5.computer---> (public IP)

6.username---> (Administrator)

7.password----> (paste password)

8.connect

3.Install webserver and create a webpage on windows server

1.search server manager and click

2.add roles and features

3.next

4.role based or features based installation

5.next

6.select a server from server pool

7.next

8.select one file------> webserver IIS------add features-------next------next-----next-----

install-----> close

9.click on file manager

10.click on c drive

11.inetpub-----> wwwroot ------> 2files (delete these files)

12.create a index.html file but open the notepad ---> save as index.html ---> all files

"HELLO WORLD"

4.(public IP) ----------------------->check the web

Create a Instance - amazon Linux & get the access

1.create a instance

- name ------------> amazon Linux

- instance type --> t3. micro

- key name -------> amazon-linux

- vpc ---------------> default

- sg -----------------> ssh(22) / http(80)

- create a instance

2.Get access of instance

- open the powershell terminal on the windows

- Run the command

- chmod 600 amazon-linux.pem

- ssh -i “<key_name>” ec2-user@<public_ip>

ssh -i “amazon-linux.pem” ec2-user@<public_ip>

Note:

- ssh → connect command

- -i → identity file (key pair)

- amazon-linux.pem → your private key file

- ec2-user → default user for Amazon Linux

- <Public-IP> → instance public IP

What is private ip and public ip

1.Public ip

- Public IP is assigned by the ISP (internet service provider)

- It is a globally unique (no other devices on the internet having the same public IP).

- It is routable over the internet

- It is a paid

2.Private ip

- Private IP is Assigned by system admin / router / DHCP inside the LAN.

- Locally unique only (can be repeated in different networks).

- Not routable over the internet directly (requires NAT to communicate outside).

- Free of cost (anyone can use private IP ranges).